Strategy 1: Data Augmentation

We applied data augmentation to the training dataset in order to increase the diversity of training samples and improve model generalization. The following transformations were applied to the training images:

- Resize images to 64×20064 \times 20064×200 to standardize the input size.

- RandomRotation (±5) to simulate slight variations in plate orientation.

- RandomAffine translation to mimic small camera shifts and positional changes.

- ColorJitter to simulate variations in lighting and contrast conditions.

- RandomPerspective transformation to emulate changes in camera angle and perspective distortion.

- RandomGrayscale to simulate different lighting environments or camera sensors.

These transformations generate slightly different versions of the same license plate images during training, helping the model become more robust to real-world variations such as camera angle, lighting conditions, and plate positioning.

Using this data augmentation strategy, the Character Error Rate (CER) improved from approximately 0.60 ± 0.03 in the baseline model to about 0.42 ± 0.02.